Spring 2020 Name: David Pace	ID:

# CSCI 3301 Homework # 1

Due: Tuesday March 09, 2020 (11:59 pm), via Moodle.

The rules:

- All work must be your own. You are not to work in teams on this assignment. You are not to use materials from previous offerings of this course.

- Format: Submit as a single file (via moodle) containing a PDF file. Email me (ayn@cs.uno.edu) assignment only if moodle is not working.

- You may use the textbook and lecture notes, but do NOT search the Internet for solutions.

- The submission deadline is strict. Therefore, please submit on time.

# Total Marks = 100

- [2 x 4=8 points]

- For the following C statement, what is the corresponding MIPS assembly code? Assume that the variables f, g, h, and i are given and could be considered 32-bit integers as declared in a C program. Use a minimal number of MIPS assembly instructions.

f = g + (h – 5);

addi t0, $s1, -5 # $s1 = h

add $t0, $t0, $s0 # $s0 = g

- For the following MIPS assembly instructions below, what will be the corresponding possible minimal Java/C statement?

add f, g, h

add f, i, f

int f,g,h,i;

int f = g+h;

int f = i+f;

- [3 × 5=15 points] Assume the following register contents:

$t0 = 0xAAAAAAAA, $t1 = 0x12345678

  - For the register values shown above, what is the value of $t2 for the following sequence of instructions?

sll $t2, $t0, 4 or $t2, $t2, $t1

0xbabefef8

  - For the register values shown above, what is the value of $t2 for the following sequence of instructions?

sll $t2, $t0, 4 andi $t2, $t2, –1

0xaaaaaaa0

  - For the register values shown above, what is the value of $t2 for the following sequence of instructions?

srl $t2, $t0, 3

andi $t2, $t2, 0xFFFFFFFF

0xF555555

- [15 points]

Assume $t0 holds the value 0x00101000. What is the value of $t2 after the following instructions? Explain.

slt $t2, $zero, $t0 bne $t2, $zero, ELSE j DONE

ELSE: addi $t2, $t2, 2 DONE:

$t2 = 0x101002

- [3 x 3 = 9 points] DO NOT use calculator or converters from the Internet. Show your calculation results step by step. Note that 0x means HEX.

- Translate 0xabcdef12 into binary.

1010 1011 1100 1101 1111 0001 0010

- Translate 0xabcdef12 into decimal.

2882400018

- Translate 8985 into Hex.

2319

- [2 x 7 = 14 points]

  - Provide the type, assembly language instruction, and binary representation of instruction described by the following MIPS fields:

# op=0, rs=3, rt=2, rd=3, shamt=0, funct=34

| Type | instr | op | rs | rt | rd | shamt | funct |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R-format | add | 000000 | 00011 | 00010 | 00011 | 00000 | 010000 |

  - Provide the type, assembly language instruction, and binary representation of instruction

described by the following MIPS fields:

# op=0x23, rs=1, rt=2, const=0x4

| Type | instr | op | rs | rt | const |
| --- | --- | --- | --- | --- | --- |
| I-format | addi | 100011 | 00001 | 00010 | 00100 |

- [3 × 7 =21 points] Consider the following MIPS loop:

LOOP: slt $t2, $zero, $t1

beq $t2, $zero, DONE addi $t1, $t1, -1

addi $s2, $s2, 2 j LOOP

DONE:

  - Assume that the register $t1 is initialized to the value 10. What is the value in register $s2 assuming the $s2 is initially zero?

20

  - For each of the loops above, write the equivalent Java / C code routine. Assume that the registers $s1, $s2, $t1, and $t2 are integers A, B, i, and temp, respectively.

Int b = 0;

for(int i = 10; 0 < i;  i-- ) {

b += 2;
}

  - For the loops written in MIPS assembly above, assume that the register $t1 is initialized to the value N. How many MIPS instructions are executed? 5N

- [3 × 6 =18 points] Assume that for a given program 70% of the executed instructions are arithmetic, 10% are load/store, and 20% are branch.

  - Given the instruction mix and the assumption that an arithmetic instruction requires 2 cycles, a load/store instruction takes 6 cycles, and a branch instruction takes 3 cycles, find the average CPI.

CPI=cyclesinstructions=11100=0.11

  - For a 25% improvement in performance, how many cycles, on average, may an arithmetic instruction take if load/store and branch instructions are not improved at all?

2-2*.25= 1.5 cycles

  - For a 50% improvement in performance, how many cycles, on average, may an arithmetic instruction take if load/store and branch instructions are not improved at all?

2-2*.50= 1.0 cycle